Supporting Information

Nb2CTx MXene : High Capacity and Ultra-long Cycle Capability for Lithium-Ion Battery by Regulation of Functional Groups

Jiabao Zhaoa, Jing Wena, Junpeng Xiaoa, Xinzhi Maa, Jiahui Gaoa, Lina Baia, Hong Gaoa, *, Xitian Zhanga, Zhiguo Zhangb

a Key Laboratory for Photonic and Electronic Bandgap Materials, Ministry of Education, School of Physics and Electronic Engineering, Harbin Normal University, Harbin 150025, Heilongjiang, China

b School of Instrumentation Science and Engineering, Harbin Institute of Technology, Harbin 150001, Heilongjiang, China

Table S1. XPS peak fitting results for O 1s region of various Nb2CTx samples.

| Sample | BE [eV] | Assigned to | Reference |
| --- | --- | --- | --- |
| Nb2CTx | 530.10 | NbOx | [1] |
|  | 531.00 | Nb2COx | [2] |
|  | 532.30 | Nb2C(OH) x | [3] |
|  | 533.50 | H2O | [4] |
| Li-Nb2CTx | 529.15 | Nb2COLix | [5] |
|  | 529.95 | NbOx | [1] |
|  | 530.80 | Nb2COx | [2] |
|  | 532.20 | Nb2C(OH) x | [3] |
|  | 533.50 | H2O | [4] |
| Li-Nb2CTx-200 | 529.20 | Nb2COLix | [5] |
|  | 530.00 | NbOx | [1] |
|  | 530.80 | Nb2COx | [2] |
|  | 532.20 | Nb2C(OH)x | [3] |
| Li-Nb2CTx-300 | 529.30 | Nb2COLix | [5] |
|  | 530.10 | NbOx | [1] |
|  | 530.80 | Nb2COx | [2] |
|  | 532.20 | Nb2C(OH) x | [3] |
| Li-Nb2CTx-400 | 529.33 | Nb2COLix | [5] |
|  | 530.13 | NbOx | [1] |
|  | 530.80 | Nb2COx | [2] |
|  | 532.20 | Nb2C(OH) x | [3] |
| Li-Nb2CTx-500 | 529.35 | Nb2COLix | [5] |
|  | 530.17 | NbOx | [1] |
|  | 530.80 | Nb2COx | [2] |
|  | 532.20 | Nb2C(OH) x | [3] |
| Nb2CTx-400 | 530.13 | NbOx | [1] |
|  | 530.80 | Nb2COx | [2] |
|  | 532.20 | Nb2C(OH) x | [3] |

Table S2. XPS peak fitting results for F 1s region of various Nb2CTx samples.

| Sample | BE [eV] | Assigned to | Reference |
| --- | --- | --- | --- |
| Nb2CTx | 684.45 | Nb2CFx | [6] |
| Li-Nb2CTx | 684.50 | Nb2CFx | [6] |
|  | 685.47 | LiF | [7] |
| Li-Nb2CTx-200 | 684.50 | Nb2CFx | [6] |
|  | 685.49 | LiF | [7] |
| Li-Nb2CTx-300 | 684.50 | Nb2CFx | [6] |
|  | 685.50 | LiF | [7] |
| Li-Nb2CTx-400 | 684.50 | Nb2CFx | [6] |
|  | 685.51 | LiF | [7] |
| Li-Nb2CTx-500 | 684.50 | Nb2CFx | [6] |
|  | 685.53 | LiF | [7] |
| Nb2CTx-400 | 684.50 | Nb2CFx | [6] |

Table S3 shows the content of O element and its constituents in each sample in XPS. The content of −O, −OH, NbOx, O-Li and H2O in the table is obtained by quantitative analysis of the area of each peak under O 1s in XPS.

Table S3. The content of O element and its constituents in each sample in XPS.

| sample/content | O 1s | -O | -OH | NbOx | O-Li | H2O |
| --- | --- | --- | --- | --- | --- | --- |
| Nb2CTx | 29.7% | 50.0% | 21.8% | 21.2% | 0.00% | 7.0% |
| Li-Nb2CTx | 43.8% | 35. 6% | 28.4% | 26.0% | 6.1% | 4.0% |
| Li-Nb2CTx-200 | 41.8% | 37.4% | 29.6% | 27.9% | 5.1% | 0.0% |
| Li-Nb2CTx-300 | 45.7% | 38.7% | 22.5% | 30.6% | 8.6% | 0.0% |
| Li-Nb2CTx-400 | 47.5% | 60.0% | 5.5% | 29.5% | 5.0% | 0.0% |
| Li-Nb2CTx-500 | 46.0% | 45.1% | 5.6% | 43.0% | 6.3% | 0.0% |
| Nb2CTx-400 | 32.8% | 61.6% | 8.2 % | 30.2% | 0.0% | 0.0% |

Table S4. The content of F element and its constituents in each sample in XPS.

| sample/content | F 1s | -F | Li-F |
| --- | --- | --- | --- |
| Nb2CTx | 10.6% | 100% | 0 |
| Li-Nb2CTx | 4.5% | 70.7% | 29.4% |
| Li-Nb2CTx-200 | 6.2% | 51.0% | 49.0% |
| Li-Nb2CTx-300 | 3.5% | 74.0% | 26.0% |
| Li-Nb2CTx-400 | 2.0% | 35.8% | 64.3% |
| Li-Nb2CTx-500 | 1.7% | 39.3% | 60.7% |
| Nb2CTx-400 | 3.1% | 100% | 0 |

Fig. S1. (a) Survey XPS spectrum, (b) O 1s spectra, (c) F 1s spectra of Li-Nb2CTx-200, Li-Nb2CTx-300 and Li-Nb2CTx-500.

Fig. S2. The Li 1s spectrum of (a) Nb2CTx and (b) Li-Nb2CTx in XPS.

For the Nb2CTx, the (002) peak of untreated Nb2CTx after 400 ℃ annealing is shifted to a larger angle with interlayer spacing about 7.62 Å due to the removal of water between layers (Fig. S3a). Meanwhile, the SEM images of untreated Nb2CTx annealed at 400 is shown in Fig. S3b, and Nb2CTx-400 maintains a good layered structure. O− and F−high resolution spectra are shown in Fig. S3c and d, respectively. The contents of −O ,−OH and −F functional groups of Nb2CTx-400 are 20.2%, 2.7% and 3.1%, respectively. As shown in Fig. S3e, at a current density of 0.05 A g-1, the specific capacity of Nb2CTx-400 can reach 257 mAh g-1 after 50 cycles. Nb2CTx-400 exhibits stable and durable capacity at different current densities (0.05~2A g-1) and decreases steadily as current density increases (Fig. S3f). At the same time, the ultra-long cycle stability of Nb2CTx-400 electrode under high current density of 2 A g-1 was tested, too. After the ultra-long cycle of 2000 cycle, the specific capacity was maintained at 186 mAh g-1 (Fig. S3g).

Fig. S3. (a) XRD, (b) SEM, (c) O 1s spectra, (d) F 1s spectra, (e) Cycling properties at 0.05 A g-1, (f) Rate performance and (g) Cycling performance at 2.0 A g-1 of Nb2CTx-400.

Fig. S4. CV curves of (a) Nb2CTx, (b) Li-Nb2CTx, (c) Li-Nb2CTx-200, (d) Li-Nb2CTx-300, (e) Li-Nb2CTx-500 and (f) Nb2CTx-400 at 0.5 mV s-1.

Fig. S5. The corresponding coulombic efficiency of different Nb2CTx MXene at 0.05 A g-1.

References

- T.M. Su, R. Peng, Z.D. Hood, M. Naguib, I.N. Ivanov, J.K. Keum, Z.Z. Qin, Z.H. Guo, Z.L. Wu, ChemSusChem 11 (2018) 688−699.

- H.R. Byon, B.M. Gallant, S. Woo and Y. Shao-Horn, Adv. Funct. Mater. 23 (2013) 1037−1045.

- J. Halim, K.M. Cook, M. Naguib, P. Eklund, Y. Gogotsi, J. Rosen and M.W. Barsoum, Appl. Surf. Sci. 362 (2016) 406−417.

- J. Halim, S. Kota, M. R. Lukatskaya, M. Naguib, M. Q. Zhao, E. J. Moon, J. Pitock, J. Nanda, S. J. May, Y. Gogotsi and M. W. Barsoum, Adv. Funct. Mater. 26 (2016) 3118.

- B. Heim, C.N. Taylor, D.M. Zigon, S. O'Dell, J.P. Allain, Nucl. Instrum. Meth. B. 269 (2011) 1262−1265.

- Y.D. Agnese, M.R. Lukatskaya, K.M. Cook, P.L. Taberna, Y. Gogotsi, P. Simon, Electrochem. Commun. 48 (2014) 118−122.

- P. Niehoff, S. Passerini, M. Winter, Langmuir 29 (2013) 5806−5816.